SAVEETHA SCHOOL OF ENGINEERING
TRANSMISSION LINES & WAVEGUIDES – ECA15
EXPERIMENT – 1

| NAME: | _____YOGESWARAN K_______________________ |
| --- | --- |
| REG. NO.: | _______192412615_____________________ |

Estimation of Characteristic Impedance (Z₀) and Propagation Constant (γ) of a Transmission Line from Primary Constants

Software Required: SCILAB Version 6.1.1

Calculation used in the manual:
Z₀ = √[(R + jωL)/(G + jωC)]
γ = √[(R + jωL)(G + jωC)] = α + jβ

Source note: The manual's printed SCILAB program uses C=16 µF/km, while the Experiment 1 test-case statement and output table specify C=8 µF/km. The completed tables below use the stated test-case values (C=8 µF/km and G=4 µmho/km) rather than silently changing the experiment data.

TEST CASE 1

Investigating the significance of Characteristic Impedance (Z₀) and Propagation Constant (γ) using R=10 Ω/km, C=8 µF/km, L=3.5 mH/km, G=4 µmho/km and f=2 kHz.

INPUTS: R = 10 Ω/km, L = 3.5 mH/km, C = 8 µF/km, G = 4 µmho/km, f = 2 kHz.

PROGRAM:

clear; clc;
R=10;L=0.003;f=2*10^3;G=4*(10^-6);C=16*(10^-6);
w=2*%pi*f;
Z=R+(%i*w*L);
Y=G+(%i*w*C);
Zo=sqrt(Z/Y);
C=round(real(Zo));
D=round(imag(Zo));
printf('-Zo = %f + j(%f)ohms\n',C,D);
P=sqrt(Z*Y);
a=real(P);
a1=round(a*10000)/10000;
printf('-Attenuation constant a = %f neper/km\n',a1);
b=imag(P);
b1=round(b*10000)/10000;
printf('-Phase constant b = %f radians/km',b1);

OUTPUT: Z₀ = 21.0496 - j2.3624 Ω; α = 0.2376 neper/km; β = 2.1161 radians/km.

| S. No. | Z₀ (Ω) | α (neper/km) | β (radians/km) | Theoretical α | Theoretical β |
| --- | --- | --- | --- | --- | --- |
| 1 | 21.0496 - j2.3624 Ω | 0.2376 | 2.1161 | 0.2376 | 2.1161 |

RESULT: The characteristic impedance and propagation constant of the transmission line are calculated using SCILAB, and the impact of the primary constants on the secondary constants is investigated.

TEST CASE 2

Examine the impact of resistance and inductance on Z₀ and γ at f=2.5 kHz. Resistance is varied from 10 Ω to 50 Ω in steps of 10 Ω; inductance is varied from 3.5 mH to 11.5 mH in steps of 2.5 mH, with the other primary constants kept constant.

CONSTANTS USED: G = 4 µmho/km, C = 8 µF/km, f = 2.5 kHz. For the R sweep L = 3.5 mH/km. For the L sweep R = 10 Ω/km.

PROGRAM:

clear; clc;
R=10;L=0.003;f=2*10^3;G=4*(10^-6);C=16*(10^-6);
w=2*%pi*f;
Z=R+(%i*w*L);
Y=G+(%i*w*C);
Zo=sqrt(Z/Y);
C=round(real(Zo));
D=round(imag(Zo));
printf('-Zo = %f + j(%f)ohms\n',C,D);
P=sqrt(Z*Y);
a=real(P);
a1=round(a*10000)/10000;
printf('-Attenuation constant a = %f neper/km\n',a1);
b=imag(P);
b1=round(b*10000)/10000;
printf('-Phase constant b = %f radians/km',b1);

OUTPUT – VARIABLE R:

| S. No. | R (Ω/km) | Z₀ (Ω) | α (neper/km) | β (rad/km) | Theoretical α, β |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 21.0022 - j1.8942 Ω | 0.2381 | 2.6392 | 0.2381, 2.6392 |
| 2 | 20 | 21.2492 - j3.7446 Ω | 0.4707 | 2.6702 | 0.4707, 2.6702 |
| 3 | 30 | 21.6322 - j5.5177 Ω | 0.6935 | 2.7184 | 0.6935, 2.7184 |
| 4 | 40 | 22.1196 - j7.1949 Ω | 0.9042 | 2.7796 | 0.9042, 2.7796 |
| 5 | 50 | 22.6813 - j8.7710 Ω | 1.1023 | 2.8502 | 1.1023, 2.8502 |

OUTPUT – VARIABLE L:

| S. No. | L (mH/km) | Z₀ (Ω) | α (neper/km) | β (rad/km) | Theoretical α, β |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.5 | 21.0022 - j1.8942 Ω | 0.2381 | 2.6392 | 0.2381, 2.6392 |
| 2 | 6.0 | 27.4246 - j1.4504 Ω | 0.1824 | 3.4463 | 0.1824, 3.4463 |
| 3 | 8.5 | 32.6188 - j1.2193 Ω | 0.1534 | 4.0990 | 0.1534, 4.0990 |
| 4 | 11.0 | 37.0965 - j1.0720 Ω | 0.1349 | 4.6617 | 0.1349, 4.6617 |

RESULT: The impact of resistance and inductance on the characteristic impedance and propagation constant of the transmission line is examined at 2.5 kHz.

TEST CASE 3

Explore the impact of frequency and capacitance on Z₀ and γ. Frequency is varied from 2 kHz to 10 kHz in steps of 2 kHz, and capacitance from 8 µF/km to 16 µF/km in steps of 2 µF/km, with the other primary constants kept constant.

CONSTANTS USED: R = 10 Ω/km, L = 3.5 mH/km, G = 4 µmho/km. For the frequency sweep C = 8 µF/km. For the capacitance sweep f = 2.5 kHz, following the manual's output-table constants.

PROGRAM:

clear; clc;
R=10;L=0.003;f=2*10^3;G=4*(10^-6);C=16*(10^-6);
w=2*%pi*f;
Z=R+(%i*w*L);
Y=G+(%i*w*C);
Zo=sqrt(Z/Y);
C=round(real(Zo));
D=round(imag(Zo));
printf('-Zo = %f + j(%f)ohms\n',C,D);
P=sqrt(Z*Y);
a=real(P);
a1=round(a*10000)/10000;
printf('-Attenuation constant a = %f neper/km\n',a1);
b=imag(P);
b1=round(b*10000)/10000;
printf('-Phase constant b = %f radians/km',b1);

OUTPUT – VARIABLE FREQUENCY:

| S. No. | f (kHz) | Z₀ (Ω) | α (neper/km) | β (rad/km) | Theoretical α, β |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 21.0496 - j2.3624 Ω | 0.2376 | 2.1161 | 0.2376, 2.1161 |
| 2 | 4 | 20.9502 - j1.1868 Ω | 0.2387 | 4.2123 | 0.2387, 4.2123 |
| 3 | 6 | 20.9315 - j0.7919 Ω | 0.2389 | 6.3128 | 0.2389, 6.3128 |
| 4 | 8 | 20.9249 - j0.5941 Ω | 0.2390 | 8.4144 | 0.2390, 8.4144 |
| 5 | 10 | 20.9219 - j0.4754 Ω | 0.2390 | 10.5165 | 0.2390, 10.5165 |

OUTPUT – VARIABLE CAPACITANCE:

| S. No. | C (µF/km) | Z₀ (Ω) | α (neper/km) | β (rad/km) | Theoretical α, β |
| --- | --- | --- | --- | --- | --- |
| 1 | 8 | 21.0022 - j1.8942 Ω | 0.2381 | 2.6392 | 0.2381, 2.6392 |
| 2 | 10 | 18.7849 - j1.6943 Ω | 0.2662 | 2.9507 | 0.2662, 2.9507 |
| 3 | 12 | 17.1482 - j1.5467 Ω | 0.2916 | 3.2323 | 0.2916, 3.2323 |
| 4 | 14 | 15.8761 - j1.4320 Ω | 0.3150 | 3.4913 | 0.3150, 3.4913 |
| 5 | 16 | 14.8508 - j1.3395 Ω | 0.3367 | 3.7324 | 0.3367, 3.7324 |

RESULT: The impact of frequency and capacitance on the characteristic impedance and propagation constant of the transmission line is investigated for the specified frequency and capacitance variations.